Experiment Manual II

MEC-aware link selection — Random Forest estimation and GMAPPO comparison

Use this manual to execute the fixed experiments exactly as defined in the thesis. The manual specifies what to vary, what to hold fixed, what to log, what to average, and which final figures/tables must be produced. Algorithm internals are intentionally excluded.

1. Purpose and final outputs

This manual executes only the experiments required for Research Content 2. It treats the selectable object as an end-to-end service path from source buoy to terrestrial sink, not as a single hop.

Final deliverables from this manual: one Random-Forest effectiveness chart, one reward-curve chart for GMAPPO under different learning rates, and two line charts of Link Advantage under different experimental conditions.

All path metrics must be built from the same common PHY quantities used in the thesis.

2. Locked formulas for this manual

Keep the following definitions identical to the thesis. P_sig,ij(t) = P_tx,j(t) · G_tx,j · G_rx,i · 10^(-PL_ij(t)/10) · ¦g_ij(t)¦^2 RSSI_ij(t) = P_sig,ij(t) + I_i(t) + N_env,i(t) SNR_ij(t) = P_sig,ij(t) / N_env,i(t), SINR_ij(t) = P_sig,ij(t) / (I_i(t) + N_env,i(t)) Q_ij(t) = PRRhat_ij(t) LET_ij(t) = +∞ if ¦¦Δv_ij¦¦ = 0 and d_ij < R_comm, else the positive root of ¦¦Δp_ij + τΔv_ij¦¦² = R_comm² P_surv,ij(t;τ_req) ≈ ∏_{n=1}^{N_p} Φ((μ_gamma,ij(t+n) − gamma_ho) / sigma_gamma,ij(t+n)) S_HO,ij(t) = (1 − exp(−LET_ij(t)/τ_req)) · P_surv,ij(t;τ_req) Q_pi(t) = (∏_{(u,v)∈π} Q_uv(t))^(1/¦π¦), S_pi(t) = (∏_{(u,v)∈π} S_HO,uv(t))^(1/¦π¦) LA_pi(t) = w_Q · Q_pi(t) + w_S · S_pi(t), w_Q + w_S = 1

3. Fixed setup before execution

| Item | Locked setting |
| --- | --- |
| Node count default | N_total = 120 unless the experiment varies it. |
| Node-count sweep | N_total ∈ {40, 80, 120, 160, 200}; class fractions remain fixed. |
| Channel-condition sweep | eta_ch ∈ {0.75, 1.0, 1.25, 1.5}; node mobility remains active. |
| Candidate-neighbor cap | K_nbr = 8 after received-power prefilter. |
| Probe dataset size | N_probe = 20,000 packets per channel regime; acceptable range 10k–50k. |
| Probe packet length | L_pkt = 1024 bit. |
| Path weights | w_Q = 0.6, w_S = 0.4 unless the optional ablation is run. |
| Switching penalty | eta_sw = 0.05. |
| Learning-rate sweep | {1e-4, 3e-4, 1e-3}. |
| Training simplifications | Use two GCN layers; enumerate only feasible local choices and at most the top K_sat relay combinations. |

4. Required logs for every run

Per probe sample: RSSI, SNR, SINR, LQI, rolling means/variances, Doppler, pair type, dwell history, ACK count, probe count, empirical PRR label.

Per evaluation window: Q_pi, S_pi, LA_pi, number of switches, number of outages, number of invalid actions, selected path ID, and active node count.

Per episode for GMAPPO: mean reward, convergence step, mean LA_pi, switch count, outage count.

5. Experiment block P — offline Random Forest estimator stage

Run the environment in probe mode with the policy frozen. No GMAPPO optimization is active in this stage.

Collect labeled probe samples separately across the intended channel regimes and link classes.

Form training vectors x_k = [RSSI, SNR, SINR, LQI, rolling means and variances, Doppler, pair type, dwell history] and labels y_k = PRR_emp,k.

Train one Random Forest regressor per link class, then calibrate each with isotonic regression so outputs stay probability-like in [0, 1].

Validate using blocked K-fold splits over time and across link classes to avoid temporal leakage.

Export one effectiveness figure based on validation predictions.

| Output | Required figure design | Recommended x-axis | Recommended y-axis | Purpose |
| --- | --- | --- | --- | --- |
| Figure 1 | Metric correlation / effectiveness chart | Measured PRR_emp or feature-importance ranking | Predicted PRRhat or importance score | Demonstrate that the RF-based link-quality estimator is predictive and not random |

For thesis clarity, the most convincing chart is predicted PRRhat versus measured PRR_emp with a diagonal reference line and the validation R²/MAE shown in the caption.

You may add a secondary feature-importance bar chart in the appendix, but the required main output is a single effectiveness chart.

6. Experiment block A — GMAPPO learning-rate sweep

Fix N_total = 120, fix eta_ch = 1.0, and keep the same probe-trained RF estimator for all runs.

Train GMAPPO at learning rates {1e-4, 3e-4, 1e-3}.

Keep path masks, GCN depth, replay settings, and candidate-path generation identical.

Log the mean episodic reward and plot one reward-curve chart.

| Output | Required figure design | X-axis | Y-axis | Legend |
| --- | --- | --- | --- | --- |
| Figure 2 | Reward value curve comparison | Training episode | Mean episodic reward | lr=1e-4 / 3e-4 / 1e-3 |

7. Experiment block B — algorithm comparison under node-count variation

Fix eta_ch = 1.0 and sweep N_total over {40, 80, 120, 160, 200}.

For each node count, run GMAPPO, MAPPO, Greedy, ACO, and GA.

Use the same trained RF estimator pipeline and the same candidate-neighbor cap K_nbr = 8.

For each evaluation window, compute LA_pi from the path-level geometric means Q_pi and S_pi.

Aggregate mean window-averaged LA_pi across seeds and plot one line chart.

| Output | Required figure design | X-axis | Y-axis | Legend |
| --- | --- | --- | --- | --- |
| Figure 3 | Line chart | Total node count N_total | Mean Link Advantage LA_pi | All 5 algorithms |

8. Experiment block C — algorithm comparison under channel-condition variation

Fix N_total = 120 while nodes continue to move dynamically.

Sweep eta_ch over {0.75, 1.0, 1.25, 1.5}.

Adjust channel parameters such as shadowing spread, K-factor, and attenuation margin through eta_ch, but do not change the channel-model family.

Run GMAPPO, MAPPO, Greedy, ACO, and GA and compute mean LA_pi across windows and seeds.

| Output | Required figure design | X-axis | Y-axis | Legend |
| --- | --- | --- | --- | --- |
| Figure 4 | Line chart | Channel-condition scale eta_ch | Mean Link Advantage LA_pi | All 5 algorithms |

9. Output formatting rules

Figure 1 must directly test estimator effectiveness. Preferred main caption wording: “Validation agreement between RF-predicted PRRhat and measured PRR_emp.”

Figures 3 and 4 must use the same y-axis range whenever possible so node-count and channel-condition effects are visually comparable.

Use the filenames: Fig1_RC2_RF_effectiveness.png, Fig2_RC2_reward_lr.png, Fig3_RC2_LA_vs_nodes.png, Fig4_RC2_LA_vs_channel.png.

10. Optional ablation (appendix only)

You may additionally vary w_Q / w_S in {(0.4,0.6), (0.6,0.4), (0.8,0.2)} and eta_sw in {0.01, 0.05, 0.1, 0.2} to show the trade-off between path quality and path stability.

Do not mix this ablation into the main figures requested above.