Experiment Manual I

Neighbor Discovery (INDP) — mechanism comparison and optimization comparison

Use this manual to execute the fixed experiments exactly as defined in the thesis. The manual specifies what to vary, what to hold fixed, what to log, what to average, and which final figures/tables must be produced. Algorithm internals are intentionally excluded.

1. Purpose and final outputs

This manual executes only the experiments required for Research Content 1. It does not redesign INDP, Disco, ALOHA, IPPO, or any baseline algorithm.

Final deliverables from this manual: two line charts for neighbor-discovery accuracy, one reward-curve chart for Improved IPPO (IPPO + Global Critic) under different learning rates, two line charts for neighbor-discovery energy consumption under algorithm comparison, and one final result table.

All figures must be generated from Monte Carlo averages under the locked Chapter-1 formulas and ranges.

2. Thesis-wide formula lock for this manual

Use exactly these common PHY-layer definitions throughout the experiment campaign. P_sig,ij(t) = P_tx,j(t) · G_tx,j · G_rx,i · 10^(-PL_ij(t)/10) · ¦g_ij(t)¦^2 RSSI_ij(t) = P_sig,ij(t) + I_i(t) + N_env,i(t) SNR_ij(t) = P_sig,ij(t) / N_env,i(t) SINR_ij(t) = P_sig,ij(t) / ( I_i(t) + N_env,i(t) ) R_comm,ij = sup { d : E[SINR_ij(d)] ≥ gamma_link } F1_topo(w) = 2TP / (2TP + FP + FN) E_ND,i(w) = Σ_m [ chi_tx,i P_tx,i + chi_rx,i(P_listen,i + chi_dec,i P_dec,i) + chi_sleep,i P_sleep,i ] Δt + E_SIC,i(w)

3. Fixed simulation setup before any experiment

| Item | Locked setting |
| --- | --- |
| Environment size | Use the fixed integrated air–land–sea–space environment defined in the thesis; default total node count N_total = 120 unless the experiment varies it. |
| Mobility update interval | Δt_sim = 20 ms. |
| Discovery window | T_w = 100 ms, Δt_slot = 2 ms, N_slot = 50. |
| Noise scale default | eta_N = 1.0 unless the experiment varies it. |
| Node-count sweep | N_total ∈ {40, 80, 120, 160, 200}; class fractions stay fixed across runs. |
| Noise sweep | eta_N ∈ {0.5, 0.75, 1.0, 1.25, 1.5, 2.0}. |
| CFAR / SIC | P_fa = 10^-4, beta_SIC = 0.05, K_max = 2. |
| Topology edge rule | gamma_link = 3 dB, rho_min = 0.6, T_min = 30 ms. |
| Counterfactual efficiency | Evaluate exact difference rewards only for active transmitters. Use minibatch B_cf = 24 when the active set is large. |
| Monte Carlo repetitions | Use at least 20 seeds for mechanism comparison and at least 5 seeds for learning-rate reward curves. 10 seeds is a practical default for algorithm comparison. |

4. Required logs for every run

Run ID, random seed, experiment ID, controlled variable value, fixed settings snapshot, and algorithm/mechanism name.

Per-window TP, FP, FN, F1_topo(w), mean E_ND(w), and success/failure flags.

For optimization runs: episode index, mean reward, mean F1_topo, mean E_ND, learning rate, and wall-clock time.

Save one CSV per seed plus one merged summary CSV per experiment block.

5. Experiment block A — mechanism comparison under noise variation

Fix N_total = 120 and keep class fractions unchanged across all runs.

Sweep eta_N over {0.5, 0.75, 1.0, 1.25, 1.5, 2.0}.

For each eta_N, run INDP, Disco, and ALOHA under identical seeds.

For each seed, average F1_topo over the full evaluation horizon after the warm-up period.

Aggregate across seeds and compute mean ± standard deviation for each mechanism at each eta_N.

| Output | Required figure/table design | X-axis | Y-axis | Legend |
| --- | --- | --- | --- | --- |
| Figure 1 | Line chart | Noise scale eta_N | Mean F1_topo | INDP / Disco / ALOHA |
| Table 1A | Result table | eta_N rows | Mean F1_topo and std | One column per mechanism |

6. Experiment block B — mechanism comparison under node-count variation

Fix eta_N = 1.0.

Sweep N_total over {40, 80, 120, 160, 200}.

For each N_total, sample the node-type composition from fixed class fractions or deterministically round from the same fractions.

Run INDP, Disco, and ALOHA under identical seeds.

Average F1_topo over windows, then aggregate across seeds.

| Output | Required figure/table design | X-axis | Y-axis | Legend |
| --- | --- | --- | --- | --- |
| Figure 2 | Line chart | Total node count N_total | Mean F1_topo | INDP / Disco / ALOHA |
| Table 1B | Append to same table block or separate sheet | N_total rows | Mean F1_topo and std | One column per mechanism |

7. Experiment block C — learning-rate sweep for Improved IPPO

Fix eta_N = 1.0 and N_total = 120.

Train Improved IPPO under learning rates {1e-4, 3e-4, 1e-3}.

Keep network structure, replay strategy, horizon, and batch settings identical.

Log the per-episode mean reward. Smooth only for visualization, but preserve raw reward logs for the appendix.

Plot the reward curve versus training episode for all three learning rates on one chart.

| Output | Required figure design | X-axis | Y-axis | Legend |
| --- | --- | --- | --- | --- |
| Figure 3 | Reward value curve comparison | Training episode | Mean episodic reward | lr=1e-4 / 3e-4 / 1e-3 |

8. Experiment block D — algorithm comparison under noise variation

Fix N_total = 120 and keep all training settings identical except for the optimization algorithm.

Sweep eta_N over {0.5, 0.75, 1.0, 1.25, 1.5, 2.0}.

Compare Improved IPPO, original IPPO, Greedy, ACO, and GA.

For each run, evaluate mean neighbor-discovery energy E_ND across the same horizon used in block A.

Aggregate by seed and plot only energy, because the requested output figure for this block is energy comparison.

| Output | Required figure design | X-axis | Y-axis | Legend |
| --- | --- | --- | --- | --- |
| Figure 4 | Line chart | Noise scale eta_N | Mean E_ND (J) | Improved IPPO / IPPO / Greedy / ACO / GA |

9. Experiment block E — algorithm comparison under node-count variation

Fix eta_N = 1.0.

Sweep N_total over {40, 80, 120, 160, 200}.

Use the same algorithm set as in block D.

Compute the mean E_ND per run and aggregate across seeds.

| Output | Required figure design | X-axis | Y-axis | Legend |
| --- | --- | --- | --- | --- |
| Figure 5 | Line chart | Total node count N_total | Mean E_ND (J) | Improved IPPO / IPPO / Greedy / ACO / GA |

10. Final result table template

| Experiment | Controlled variable | Algorithms / mechanisms | Primary metric | Report format |
| --- | --- | --- | --- | --- |
| 1A | eta_N | INDP / Disco / ALOHA | F1_topo | mean ± std |
| 1B | N_total | INDP / Disco / ALOHA | F1_topo | mean ± std |
| 1D | eta_N | Improved IPPO / IPPO / Greedy / ACO / GA | E_ND | mean ± std |
| 1E | N_total | Improved IPPO / IPPO / Greedy / ACO / GA | E_ND | mean ± std |

11. Figure naming and export rules

Use PNG or PDF export with a consistent naming rule: Fig1_RC1_accuracy_vs_noise.png, Fig2_RC1_accuracy_vs_nodes.png, Fig3_RC1_reward_lr.png, Fig4_RC1_energy_vs_noise.png, Fig5_RC1_energy_vs_nodes.png.

Use line charts with markers; x-axis values must remain the true sweep values rather than rank indices.

Report mean curves; show standard-deviation shading if space permits. For a thesis, error bars are sufficient if shading becomes cluttered.

Keep the y-axis unit explicit: F1_topo is unitless, E_ND is in J.